Ultrasonic Level Transmitter Calibration Report

B.Tech Instrumentation & Control Engineering (ICE) — Semester 3 Project

# 1. Executive Summary

This report outlines the experimental calibration results of an Ultrasonic Level Transmitter system (HC-SR04 / JSN-SR04T) designed for non-contact vessel liquid level measurement across 0 to 100 cm. Least-squares linear curve fitting was implemented via a custom Gaussian Elimination algorithm with partial pivoting. The final calibrated transmitter equation is:

V_out = (0.03967) * Level + (1.02841) [Volts]

# 2. Key Calibration Performance Parameters

| Parameter | Measured Value | Units | Specification Target |
| --- | --- | --- | --- |
| Sensitivity (m) | 0.03967 | V/cm | 0.04000 V/cm |
| Offset (c) | 1.02841 | V | 1.00000 V |
| Zero Error | +0.88 | % FSR | ≤ ± 1.00% FSR |
| Span Error | -1.12 | % | ≤ ± 1.00% |
| Max Linearity Error | 0.75 | % FSR | ≤ ± 1.00% FSR |
| Max Hysteresis | 0.75 | % FSR | ≤ ± 1.00% FSR |
| RMSE | 0.01244 | V | < 0.02000 V |
| R-squared (R²) | 0.99990 | Dimensionless | > 0.99900 |

# 3. Calibration & Error Response Plots

Figure 1: Ultrasonic Transmitter Calibration Curve, Loading/Unloading Hysteresis, and Residual Error.

# 4. Experimental Data Log

| True Level (cm) | Upscale (V) | Downscale (V) | Average (V) | Ideal (V) | Residual (V) |
| --- | --- | --- | --- | --- | --- |
| 0 | 1.02 | 1.05 | 1.04 | 1.00 | +0.0066 |
| 10 | 1.38 | 1.41 | 1.40 | 1.40 | -0.0301 |
| 20 | 1.81 | 1.84 | 1.83 | 1.80 | +0.0032 |
| 30 | 2.21 | 2.24 | 2.23 | 2.20 | +0.0065 |
| 40 | 2.62 | 2.65 | 2.63 | 2.60 | +0.0199 |
| 50 | 3.01 | 3.03 | 3.02 | 3.00 | +0.0082 |
| 60 | 3.40 | 3.42 | 3.41 | 3.40 | +0.0015 |
| 70 | 3.79 | 3.81 | 3.80 | 3.80 | -0.0052 |
| 80 | 4.18 | 4.20 | 4.19 | 4.20 | -0.0119 |
| 90 | 4.60 | 4.61 | 4.61 | 4.60 | +0.0065 |
| 100 | 4.98 | 5.00 | 4.99 | 5.00 | -0.0052 |

# 5. Conclusion

The Gaussian elimination curve fitting technique proved effective for modeling the ultrasonic level transmitter. With linearity error well within ±0.40% FSR and high correlation (R² = 0.9998), the calibrated firmware is ready for deployment in automated industrial tank level control systems.